# Subnetting exercises

- How many host addresses are available in the following networks?

| IP | Subnet mask in dotted decimal | Network bits | Host bits | # available hosts in the network |
| --- | --- | --- | --- | --- |
| 151.238.65.128/25 |  |  |  |  |
| 138.246.224.0/20 |  |  |  |  |
| 218.85.109.16/29 |  |  |  |  |
| 221.123.100.0/27 |  |  |  |  |
| 28.49.0.0/.. | 255.255.252.0 |  |  |  |
| 216.165.176.64/.. | 255.255.255.192 |  |  |  |
| 222.118.173.0/.. | 255.255.240.0 |  |  |  |
| 172.86.156.0/.. | 255.255.248.0 |  |  |  |

- Fill in the table using the given IP address

| IP | Network Address | Broadcast Address | First Host Address | Last Host Address | # available hosts |
| --- | --- | --- | --- | --- | --- |
| 197.53.96.135/28 |  |  |  |  |  |
| 182.48.180.135/24 |  |  |  |  |  |
| 1.54.237.129/21 |  |  |  |  |  |
| 12.210.139.230/25 |  |  |  |  |  |
| 111.172.197.153/17 |  |  |  |  |  |
| 194.99.221.165/27 |  |  |  |  |  |
| 72.1.7.47/22 |  |  |  |  |  |

- A router is configured with the IP address 114.135.225.17/28. Which of the following IP addresses can I use to assign to a host in that network?

| IP | Network Address | Broadcast Address | First Host Address | Last Host Address |
| --- | --- | --- | --- | --- |

- 114.135.225.17

- 114.135.225.16

- 114.135.225.18

- 114.135.225.0

- 114.135.225.1

- 114.135.225.31

- 114.135.225.30

- 114.135.225.32

- 114.135.225.64

- 114.135.225.65

- 114.135.225.254

- Subnet the following network based on the number of needed networks using the network address: 173.231.70.128/25 and address all devices

Number of needed networks: 
Number of additional network bits: 
Total number of networks: 
Number of unused networks: 
New subnet mask:

| Network | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- |

- Subnet the following network based on the number of needed networks using the network address: 199.202.128.0/17 and address all devices

Starting network address: 
Number of needed networks: 
Number of additional network bits: 
Total number of networks: 
Number of unused networks: 
New subnet mask: 
Number of hosts per network:

| Network | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- |

- Subnet the following network based on the maximum number of needed hosts using the network address: 137.65.208.0/20 and address all devices

Maximum number of hosts on a network:
Number of needed host bits:
Total number of networks:
New subnet mask: 
Number of hosts per network:

| Network | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- |