Exercise 6 - Transport and Application Layer

In this assignment we will examine the protocols working in the transport layer and the application layer. In the previous assignments,  a step by step guide was provided. In this assignment you will have to figure out yourself how to  get the information you need to answer the questions using the knowledge you obtained in the previous assignments. Don’t just lookup the information on the internet, but find the needed information using Wireshark and/or packet tracer to find the information. Even if you know the answer beforehand, try to verify the answer in Wireshark/Packet Tracer.

Some additional information is added in italic gray. Try to answer the questions without looking at this information first. If you don’t know how to start, you can read this information.

On Canvas, you can find a large text file (1Mb) that you can use to answer the questions: “largefile.txt”

# Answer the questions

## Wireshark

- What is the source and destination IP and MAC address of a DHCP request?

You can open Wireshark and start monitoring. Than disconnect and reconnect to a network to capture the DHCP packets. You can use the filter “dhcp”.

- What is the port number on which a DHCP server is listening on?

You can find the destination port number of a DHCP discover or request packet. This is the port number the DHCP server is listening on.

- What is the port number on which a DNS server is listening on?

You can start Wireshark and use the nslookup command to send a query to domain name of your choise. You can use the filter “dns”. The destination port of the standard query to the DNS server is the port on which the DNS server is listening on.

- Which transport layer protocol is used to send a standard query to a DNS server?

Using the previous capture, you can open the details of a standard DNS query. There you can find the protocol used in the transport layer.

- Which application layer protocol is used to open the website http://www.seamine.be?

You can open wireshark and visit the website. Than get the ip address of this domain name and apply this address as a filter in wireshark. You can see which application layer protocol is used.

- Which Transport layer protocol does this application layer protocol use?

Opening the HTTP Get request line in wireshark, you can see the protocols it uses in the bottom window.

- Which packets are send to and from the destination to setup and tear down a TCP connection?

Using the previous wireshark capture, you can apply the filter “ip.addr==94.231.103.72 && tcp”. The first packets are the ones used to setup the connection.

- What is the window size used by the server when downloading the largefile.txt? And how many packets can be send before your computer has to send an acknowledgment?

You can start Wireshark and download the file. You can use the ip address of seamine.be as a filter. Look for the HTTP GET request from your ip address to the webserver. The TCP packets following this get request are the segments of the largefile.txt send from the webserver to your computer. You can see several packets send from the server to your computer. And then a packet that is send from your computer to the server acknowledging the received packets (Can you figure out why that specific acknowledgment number is used?) The window size is set in every packet send from the server to your computer.

Using this window size and the size of the data packet, which can be found in the Len field of the TCP header, you can find the number of packets that can be send before an acknowledgment has to be send by your computer to the server.

- What is the port number generated by the browser tab to download the largefile.txt file.

The port number can be found in the previous capture by looking at the source port of any of the packets send from your computer to the server of the destination port of every packet send from the server to your computer.

## Packet tracer

- Opening the file “Exercise 6 - Transport and Application Layer.pkt”. Open the browser on PC0 and try to access the website http://test.be. This will not work. What is the problem/problems in this network configuration? The problems can be located in all the components of the network. Fix the problems so that the webpage is showed on PC0s browser using the test.be domain name. It is possible that there are more than one solution to one or more of the problems. Implementing one of these solutions is enough.

The best way to figure out what the problem is, is to enter the simulation mode and enable only the relevant protocols as a filter. Than open the browser and try to open test.be. Now step through the packets send over the network. Now you can see where the process fails. Fix the problem and repeat until the network does what you expect.

This way you can also examine the content of the packets to get some additional information.